How to update the firmware for the KA11

注意：在为KA11升级固件之前，请拔掉电脑上USB接口的其他解码器！

Note: Before upgrading the firmware for KA11, please unplug other decoders from the USB interface on the computer!

以下是关于KA11 V0.07固件的一些说明，请悉知：

a）升级V0.07固件后，若遇到音量小的情况，可以在Fiio Control App中调大音量。

b）在Fiio Control App中调节音量时，可能会出现单声道的情况，此时再次调节音量就可以恢复正常，不影响使用。建议在App中调节好合适的音量后，后续使用手机等播放设备自带的音量按键进行调节。

c）V0.07固件只是增加了音量调节的功能。如果自身设备的自带音量调节可以满足需求，可以不用更新固件。

The following are some instructions about the KA11 V0.07 firmware, please know:

a) After upgrading the V0.07 firmware, if you encounter a low volume, you can increase the volume in the Fiio Control App.

b) When adjusting the volume in the Fiio Control App, monophonic sound may occur. At this time, adjusting the volume again can return to normal without affecting use. It is recommended that after adjusting the appropriate volume in the App, you can then use the volume buttons that come with your mobile phone or other playback device to adjust it.

c) The V0.07 firmware only adds the volume adjustment function. If the built-in volume adjustment of your device can meet your needs, you don't need to update the firmware.

- 安装Windows驱动：https://bbs.fiio.com/note/showNoteContent.do?id=201911191752415983705&tid=16

- Install the Windows driver: https://forum.fiio.com/note/showNoteContent.do?id=202105191527366657910&tid=17

- 将耳机与KA11连接，KA11与Windows电脑连接。

- 2. Connect the earphone to KA11, and connect KA11 to the Windows computer.

- 双击“SA9312LWriteEepromMP_20240129_v2p0_EE_CS43131_2972_0081_FIIO_FIIOKA11_ONLY”打开烧录工具，修改对应的Vendor ID和Product ID。

- Vendor ID为2972，Product ID为0081。

- 3. Double-click "SA9312LWriteEepromMP_20240129_v2p0_EE_CS43131_2972_0081_FIIO_FIIOKA11_ONLY" to open the burning tool and modify the corresponding Vendor ID and Product ID. The Vendor ID is 2972 and the Product ID is 0081.

- 点击“Write EEPROM”进行固件烧录，当出现以下弹窗时，重新拔插KA11的USB接口（重新上电），然后点击“确定”。

- 4. Click "Write EEPROM" to burn the firmware. When the following pop-up window appears, re-plug the USB interface of KA11 (power on again), and then click "确定".

- 5. 当出现“Write to EEPROM OK!”时，即表明固件烧录成功。

- 5. When "Write to EEPROM OK!" appears, the firmware is burned successfully.

- 6. 安装成功后，可以拔掉KA11，再连接电脑，就可以在Fiio控制面板中看到KA11已经显示为新固件版本号了。

- 6. After the installation is successful, you can unplug KA11 and connect it to the computer. You can see that KA11 has been displayed as the new firmware version number in the Fiio control panel.